Project03_46W38 – Wind Resource Assessment Based on Reanalysis Data

by

Yong Jun Cho

S253940

Technical University of Denmark

Supervised by

Ju Feng

Dec 2025

# Introduction

This project develops a Python module for performing a simplified wind resource assessment (WRA) using ERA5 reanalysis data at the Horns Rev1 offshore wind farm. The module automates core steps commonly used in preliminary wind studies:

- Wind data extraction and interpolation,

- Wind speed extrapolation to turbine hub height,

- Statistical wind climate modeling (Weibull distribution),

- AEP calculation from turbine power curves.

# Structure and Design

The project uses a src-layout Python package structure:

src/

|- wind_assess/

|- __init__.py	-> Public API

|- core.py		-> WindResource Class (main function)

|- utils.py		-> Power curve and plotting utilities

## WindResource Class (core.py)

Implements the main analysis workflow:

- load and merge ERA5 NetCDF files

- compute wind speed & direction

- spatial interpolation

- shear exponent estimation

- power-law extrapolation

- Weibull fitting

- AEP calculation

## utils.py

provides:

- power curve CSV loader

- Weibull plot

- wind rose plot

## examples/main.py

Demonstrates end to end usage and produces plots and AEP outputs.

## tests/

Contains pytest unit tests achieving 88% coverage.

# Results

Figure 1: Terminal output

## Wind conditions at Horns Rev 1

From the 1997–2008 ERA5 dataset:

- Mean wind speed at 100m: 9.6 m/s

- Prevailing wind direction: 210° (southwest)

- Weibull fit at 100 m:

  - k=2.3

  - A=10.8 m/s

Wind rose and Weibull distribution curve can be found in the output/ folder

## AEP of the testing year 2002

Table 1:AEP Result of 5MW and 15MW Turbines

| Turbine | Hub Height | AEP (MWh) |
| --- | --- | --- |
| NREL 5 MW | 90 m | 23 812 MWh |
| NREL 15 MW | 150 m | 78 352 MWh |

# Conclusion

The implemented module successfully automates the essential steps of a wind resource assessment based on ERA5 reanalysis data.

The project demonstrates:

- Correct usage of scientific Python tools (xarray, numpy, scipy)

- Reliable wind resource assessment results

- Structured software design

- Automated tests ensuring correctness

The code can be easily extended to handle:

- More turbine models

- Additional reanalysis sources

- Long-term corrections